CPE301 – SPRING 2025

# Design Assignment 5

Student Name: Carlos Funes

Student #: 5007995832

Student Email: funesc2@unlv.nevada.edu

Primary Github address: https://github.com/CaFu0320

Directory: https://github.com/CaFu0320/submission_da/tree/main/DesignAssignments/DA5

Video Playlist: https://youtube.com/playlist?list=PLa3xS6s509hhSrDtIiRlyynXMlhipu13Z&si=vMDsBcMr7TgJm_Hq

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- ATMEGA328PB MINI BOARD

- ARDUINO SHIELD

- UART

- EXTERNAL POTENTIOMETER

- DC MOTOR

- POLOLU MOTOR DRIVER CHIP

- BREADBOARD

- C CODE TASKS

/*

* DA5

* Created: 4/19/2025 10:27:07 AM

* Author : Carlos Funes

*/

#define F_CPU 16000000UL //defining CPU frequency

#include <avr/io.h> //including AVR I/O register definitions

#include <avr/interrupt.h> //including interrupt handling functions

#include <stdio.h> //including standard I/O functions for snprintf

#include <stdlib.h> //for functions

#include <util/delay.h> //for delay_ms()

//global variables

volatile uint8_t uartOverride = 0; //flag that decides if user should override ADC value

volatile uint8_t uartPWMValue = 0; //PWM value set by the user input

volatile uint32_t revCtr = 0; //number of pulses counted

char outs[72]; //buffer for messages to UART

#define PULSES_PER_REV 1 //pulses per one full motor rotation

//UART functions

void USART_Init(unsigned int baud) { //setting baud rate to 9600

uint16_t ubrr = F_CPU / 16 / baud - 1; //calculating UART speed

UBRR0H = (uint8_t)(ubrr >> 8); //setting baud rate high byte

UBRR0L = (uint8_t)(ubrr); //setting baud rate low byte

UCSR0B = (1 << RXEN0) | (1 << TXEN0); //enabling transmitter and receiver

UCSR0C = (1 << UCSZ01) | (1 << UCSZ00); //8 data bits, 1 stop bit

}

void USART_Transmit(char data) { //sending one character to UART

while (!(UCSR0A & (1 << UDRE0))); //waiting until transmit buffer is ready

UDR0 = data; //send data when ready

}

void USART_SendString(const char* str) { //sending strings to UART

while (*str) USART_Transmit(*str++); //sending string characters

}

int USART_Receive_NonBlocking(void) { //checking if computer sent something

if (UCSR0A & (1 << RXC0)) return UDR0; //return if new data is received

return -1; //otherwise, don't

}

//ADC functions

void ADC_Init() { //setting up ADC to read voltages

DDRC &= ~(1 << DDC0); //PC0 as input

ADMUX = 0x40; //using AVcc as ref voltage in channel 0

ADCSRA = 0x87; //enabling ADC and giving it a prescaler of 128

}

int ADC_Read(char channel) { //reading ADC value from given channel

ADMUX = 0x40 | (channel & 0x07); //selecting ADC channel

ADCSRA |= (1 << ADSC); //starting ADC conversion

while (!(ADCSRA & (1 << ADIF))); //waiting until conversion is complete

ADCSRA |= (1 << ADIF); //clearing interrupt flag

return ADCW; //returning ADC values

}

//PWM Setup

void PWM_Init() { //PWM output for motor control

DDRD |= (1 << DDD6); //PD6 for output

TCCR0A = (1 << WGM00) | (1 << WGM01) | (1 << COM0A1); //fast PWM mode

TCCR0B = (1 << CS00) | (1 << CS02); //prescaler of 1024

}

//motor direction control

void MotorDirectionInit() { //initializing motor control pins

DDRD |= (1 << DDD4) | (1 << DDD5); //setting PD4 and PD5 as outputs

PORTD |= (1 << PORTD5); //PD5 high

PORTD &= ~(1 << PORTD4); //PD4 low

}

//input capture setup

void InitTimer1(void) { //capturing encoder pulses

DDRB &= ~(1 << DDB0); //PB0 as input (ICP1)

PORTB |= (1 << PORTB0); //enabling pull-up resistor

TCNT1 = 0; //reset timer1 counter

TCCR1A = 0; //normal operation

TCCR1B = (1 << ICES1) | (1 << ICNC1); //capturing on rising edge and enable noise canceler

TIMSK1 = (1 << ICIE1); //enabling input capture interrupt

}

void StartTimer1(void) { //starting timer 1 without prescaler

TCCR1B |= (1 << CS10); //no prescaling, therefore timer1 counts at full speed

sei(); //enabling global interrupts

}

//ISRs

ISR(TIMER1_CAPT_vect) {

revCtr++; //incrementing pulse counter

USART_SendString("CAPTURE\r\n"); //sending CAPTURE to UART

}

//UART commands

void HandleUARTCommand() { //reading user commands

static char buffer[16]; //buffer to store incoming characters

static uint8_t idx = 0; //index for buffer

int c;

while ((c = USART_Receive_NonBlocking()) != -1) { //if something was received

if (c == '\r' || c == '\n') { //if character is enter key or newline

buffer[idx] = '\0'; //end string with a null character

if (idx > 0) { //making sure we have something typed

if (buffer[0] == 'S' || buffer[0] == 's') { //checking if command starts with S or s

uint16_t val = 0; //storing PWM value from string

uint8_t valid = 1; //assuming input is correct

for (uint8_t i = 1; i < idx; i++) { //after S, converting each character into a number

if (buffer[i] >= '0' && buffer[i] <= '9') { //if it's a digit

val = val * 10 + (buffer[i] - '0'); //build the number

} else {

valid = 0; //if its not a digit, we mark it as invalid

break;

}

}

if (valid && val <= 255) { //if number is valid and within 0-255 range

uartPWMValue = val; //saving PWM value

uartOverride = 1; //switching to UART control mode

snprintf(outs, sizeof(outs), "PWM:%u\r\n", val); //creating confirmation message

USART_SendString(outs); //sending confirmation to UART

} else {

USART_SendString("Invalid PWM!\r\n"); //error message for invalid input

}

} else if (buffer[0] == 'A' || buffer[0] == 'a') { //if command is A or a, switch to ADC control

uartOverride = 0;

USART_SendString("ADC mode\r\n"); //sending confirmation message

}

}

idx = 0; //resetting buffer ready for next command

} else if (idx < sizeof(buffer) - 1) { //if character is not Enter, store in buffer

buffer[idx++] = c; //saving received character and move buffer index

} else { //if too many characters without enter key

idx = 0; //clear buffer

USART_SendString("Cmd too long!\r\n"); //sending warning to user

}

}

}

int main(void) {

USART_Init(9600); //UART communication at 9600 baud rate

ADC_Init(); //preparing ADC to read potentiometer

PWM_Init(); //setting up PWM for motor speed control

MotorDirectionInit(); //set motor to move in forward direction

InitTimer1(); //detecting motor pulses

StartTimer1(); //being pulse capture

USART_SendString("System Ready\r\n"); //sending ready message

uint32_t lastUpdate = 0; //timer for RPM calculations

uint32_t lastRevCount = 0; //last pulse count

while (1) { //infinite loop

HandleUARTCommand(); //checking if user sent any commands

uint16_t adcVal = ADC_Read(0); //reading pot (ADC channel 0)

if (uartOverride) { //setting motor speed

OCR0A = uartPWMValue; //user given PWM if UART override is active

} else {

OCR0A = adcVal / 4; //using potentiometer (0-255) <<-- from 1024/255

}

if (++lastUpdate >= 1000) { //updating RPM reading every 1 second

lastUpdate = 0; //resetting counter

float rpm = 0;

uint32_t pulses = revCtr - lastRevCount; //how many pulses since last update

lastRevCount = revCtr; //saving current pulse count for next update

if (pulses > 0) {

rpm = (pulses / (float)PULSES_PER_REV) * 60.0; //calculating RPM

}

//creating message depending on control mode

if (uartOverride) {

snprintf(outs, sizeof(outs), "PWM:%u RPM:%.2f\r\n", OCR0A, rpm); //user given

} else {

snprintf(outs, sizeof(outs), "ADC:%u PWM:%u RPM:%.2f\r\n", adcVal, OCR0A, rpm); //potentiometer given

}

USART_SendString(outs); //sending rpm and pwm to the terminal

}

_delay_ms(1); //delay to make loop timing better

}

}

- SCHEMATIC

- SCREENSHOTS OF EACH TASK OUTPUT (

TASK 1

TASK 2

TASK 3

- SCREENSHOT OF DEMO

- VIDEO LINK FOR DEMO

Task: https://youtu.be/Ntrmp7nqn6s

- GITHUB LINK OF THIS DA

Tasks c code: https://github.com/CaFu0320/submission_da/tree/main/DesignAssignments/DA5

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Carlos Funes