CPE301 – SPRING 2025

# Design Assignment 6

Student Name: Carlos Funes

Student #: 5007995832

Student Email: funesc2@unlv.nevada.edu

Primary Github address: https://github.com/CaFu0320

Directory: https://github.com/CaFu0320/submission_da/tree/main/DesignAssignments/DA6

Video Playlist:

https://youtube.com/playlist?list=PLa3xS6s509hhSrDtIiRlyynXMlhipu13Z&si=vMDsBcMr7TgJm_Hq

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- ATMEGA328PB MINI BOARD

- UART

- SERVO MOTOR

- HC-SR04 ULTRASONIC SENSOR

- SEVEN SEG DISPLAY

- C CODE TASKS

/*

* DA6

*

* Created: 4/28/2025 11:50:07 PM

* Author: Carlos Funes

*/

#define F_CPU 16000000UL //defining CPU frequency

#include <avr/io.h> //including AVR I/O register definitions

#include <stdint.h> //integer types

#include <avr/interrupt.h> //including interrupt handling functions

#include <util/delay.h> //for delay_ms()

#include <stdio.h> //I/O functions

#include <stdlib.h> //for functions

#include <string.h> //string handling

//UART config

#define BAUDRATE 9600 //baud rate

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1) //prescaler calculation

char uart_buffer[64]; //buffer for UART messages

//function prototypes

void USART_init(void); //UART

void Servo_init(void); //servo motor

void Ultrasonic_init(void); //initialize ultrasonic sensor

void SPI_init(void); //initialize SPI for 7-segment

void USART_putstring(const char* str); //sending string via UART

uint16_t angle_to_OCR(uint16_t angle); //converting angle to PWM value

double read_distance_mm(void); //reading distance from sensor

void display_number_on_7seg(uint16_t val); //displaying number on 7-segment

void display_number_once(uint16_t val); //single display update

//UART initialization

void USART_init(void) {

UBRR0H = (uint8_t)(BAUD_PRESCALLER >> 8); //high byte of baud rate

UBRR0L = (uint8_t)(BAUD_PRESCALLER); //low byte

UCSR0B = (1 << RXEN0) | (1 << TXEN0); //enabling receiver and transmitter

UCSR0C = (1 << UCSZ01) | (1 << UCSZ00); //8-bit data, 1 stop bit, no parity

}

//sending single character via UART

void USART_send(unsigned char data) {

while (!(UCSR0A & (1 << UDRE0))); //waiting until buffer is empty

UDR0 = data; //loading data into transmit register

}

//sending string via UART

void USART_putstring(const char* str) {

while (*str) { //looping until end of string

USART_send(*str++); //sending each character

}

}

//servo motor initialization

void Servo_init(void) {

DDRB |= (1 << PB1); //setting PB1 as output (OC1A)

//configuring Timer1 for fast PWM mode

TCCR1A = (1 << COM1A1) | (1 << WGM11); //non-inverting, fast PWM

TCCR1B = (1 << WGM13) | (1 << WGM12) | (1 << CS11) | (1 << CS10); //prescaler 64

ICR1 = 4999; //PWM period for 50Hz (20ms)

}

//angle (0-180) to OCR1A value (105-610)

uint16_t angle_to_OCR(uint16_t angle) {

return (uint16_t)(105 + (angle * 505.0 / 180.0));

}

//ultrasonic sensor initialization (PD5 trigger, PE2 echo)

volatile uint16_t TimerOverflow = 0; //tracking Timer3 overflows

ISR(TIMER3_OVF_vect) { //timer3 overflow interrupt

TimerOverflow++; //increment overflow count

}

void Ultrasonic_init(void) {

DDRD |= (1 << PD5); //PD5 as trigger output

DDRE &= ~(1 << PE2); //PE2 (ICP3) as input

TCCR3A = 0; //normal timer3 operation

TIMSK3 |= (1 << TOIE3); //enabling timer3 overflow interrupt

}

//reading distance in mm

double read_distance_mm(void) {

PORTD |= (1 << PD5); //sending 10us trigger pulse

_delay_us(10);

PORTD &= ~(1 << PD5);

TCCR3B = (1 << ICES3) | (1 << CS10); //capturing rising edge, no prescaler

TIFR3 = (1 << ICF3) | (1 << TOV3); //clearing flags

TimerOverflow = 0; //resetting overflow counter

while (!(TIFR3 & (1 << ICF3))); //waiting for rising edge

TCNT3 = 0; //resetting timer

TCCR3B &= ~(1 << ICES3); //capturing falling edge

TIFR3 = (1 << ICF3) | (1 << TOV3); //clearing flags

TimerOverflow = 0;

while (!(TIFR3 & (1 << ICF3))); //waiting for falling edge

uint32_t pulse = (uint32_t)ICR3 + (65536 * TimerOverflow);

return (pulse / 58.0 / 16.0) * 10.0; //converting to mm

}

//SPI and 7 segment display PB2-PB5

#define DATA    (1<<PB3) //data pin (PB3)

#define LATCH   (1<<PB2) //latch (PB2)

#define CLOCK   (1<<PB5) //clock (PB5)

//segment patterns for 0-9

const uint8_t SEGMENT_MAP[10] = {

0xC0, 0xF9, 0xA4, 0xB0, 0x99, // 0-4

0x92, 0x82, 0xF8, 0x80, 0x90  // 5-9

};

//digit enable patterns for 4-digit display

const uint8_t DIGIT_SELECT[4] = {0xF1, 0xF2, 0xF4, 0xF8};

void SPI_init(void) {

DDRB |= (DATA | CLOCK | LATCH); //setting data/latch/clock pins as outputs

PORTB &= ~(DATA | CLOCK | LATCH); //initialize low

SPCR0 = (1 << SPE) | (1 << MSTR);  //enabling SPI communication, master mode

}

//sending number pattern via SPI

void SPI_send(uint8_t byte) {

SPDR0 = byte; //loading data

while (!(SPSR0 & (1 << SPIF))); //waiting until data is fully sent

}

//displaying 4-digit number with multiplexing

void display_number_on_7seg(uint16_t val) {

uint8_t digits[4] = {

(uint8_t)(val % 10),          //ones

(uint8_t)((val / 10) % 10),   //tens

(uint8_t)((val / 100) % 10),  //hundreds

(uint8_t)(val / 1000)         //thousands

};

for (int8_t i = 3; i >= 0; i--) { //cycling through each digit position

PORTB &= ~LATCH; //preparing to send data

SPI_send(SEGMENT_MAP[digits[i]]); //sending segment pattern

SPI_send(DIGIT_SELECT[i]); //selecting digit that lights up

PORTB |= LATCH; //latching high to update

_delay_ms(2); //display each digit for 2ms

}

}

//single display update

void display_number_once(uint16_t val) {

// Similar to above but without delay

uint8_t digits[4] = {val % 10, (val/10)%10, (val/100)%10, val/1000};

for (int8_t i = 3; i >= 0; i--) {

PORTB &= ~LATCH;

SPI_send(SEGMENT_MAP[digits[i]]);

SPI_send(DIGIT_SELECT[i]);

PORTB |= LATCH;

}

}

//main program

int main(void) {

uint16_t angle; //current servo angle

double distance, min_distance; //distance measurements

uint16_t display_min; //minimum distance to display

char angleStr[8], distStr[8]; //UART buffers

USART_init(); //starting serial communication

Servo_init(); //preparing servo motor

Ultrasonic_init(); //preparing distance sensor

SPI_init(); //preparing display

sei(); //enabling interrupts

USART_putstring("Angle (Degrees), Distance (mm)\n"); //printing

while (1) {

min_distance = 9999.0; //starting with a high minimum distance

//sweeping servo 0 to 180

for (angle = 0; angle <= 180; angle += 2) {

OCR1A = angle_to_OCR(angle); // Moving servo to current angle

distance = read_distance_mm(); //measuring distance

if (distance < min_distance) min_distance = distance; //tracking closest object

//formating and sending data via UART

dtostrf((double)angle, 3, 0, angleStr); //converting angle to text

dtostrf(distance, 6, 2, distStr); //converting distance to text

snprintf(uart_buffer, sizeof(uart_buffer),

"Angle: %s°   Distance: %s mm\n", angleStr, distStr); //sending this to the terminal

USART_putstring(uart_buffer);

display_min = (uint16_t)min_distance; //updating display with the closes distance

display_number_on_7seg(display_min);

_delay_ms(100); //waiting between steps

}

//servo 180 to 0

for (angle = 180; angle >= 0; angle -= 2) { //this is the same process from above but in reverse

OCR1A = angle_to_OCR(angle);

distance = read_distance_mm();

dtostrf((double)angle, 3, 0, angleStr);

dtostrf(distance, 6, 2, distStr);

snprintf(uart_buffer, sizeof(uart_buffer),

"Angle: %s°   Distance: %s mm\n", angleStr, distStr);

USART_putstring(uart_buffer);

display_number_on_7seg((uint16_t)distance);

_delay_ms(100);

}

}

}

- SCHEMATIC

- SCREENSHOTS OF EACH TASK OUTPUT (

TASK 1

TASK 2

Found on YouTube, but for demonstration purposes:

Clockwise:

https://youtube.com/shorts/q9tgixeA7Qo?si=ZEo5pHIBrCJsBaI4

Counterclockwise:

https://youtube.com/shorts/H0FlKaygN3o

- SCREENSHOT OF DEMO

- VIDEO LINK FOR DEMO

Task: https://youtu.be/keCZ4SwhddY

- GITHUB LINK OF THIS DA

Tasks c code: https://github.com/CaFu0320/submission_da/tree/main/DesignAssignments/DA6

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Carlos Funes