CPE301 – SPRING 2025

# Design Assignment 7

Student Name: Carlos Funes

Student #: 5007995832

Student Email: funesc2@unlv.nevada.edu

Primary Github address: https://github.com/CaFu0320

Directory: https://github.com/CaFu0320/submission_da/tree/main/DesignAssignments/DA7

Video Playlist: https://youtube.com/playlist?list=PLa3xS6s509hhSrDtIiRlyynXMlhipu13Z&si=vMDsBcMr7TgJm_Hq

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- ATMEGA328PB MINI BOARD

- UART

- SERIAL PLOTTER

- BMI160 6-DOF IMU SENSOR

- I2C OLED Display

- 2 BREADBOARDS

- C CODE TASKS

/*

* DA7_7v2

* Author: Carlos Funes

*/

#define F_CPU 16000000UL //defining CPU frequency

#include <avr/io.h> //including AVR I/O register definitions

#include <util/delay.h> //for delay_ms()

#include <stdlib.h> //for functions

#include <stdio.h> //I/O functions

#include <math.h> //include this to calculate angles

#include <avr/pgmspace.h> //storing data into flash memory

#define DISPLAY_WIDTH 128 //oled width in pixels

#define DISPLAY_HEIGHT 64 //height in pixels

#define LCD_I2C_ADR 0x3C //i2c address

#define ACCEL_SCALE 16384.0f //accelerometer sensitivity

#define GYRO_SCALE 16.4f //gyroscope sensitivity

#define FONT ssd1306oled_font //font data being stored in PROGMEM

const uint8_t special_char[][2] = { {0,0}, {0xFF,0xFF} }; //placeholder for graphics

void oled_init(void); //turning on oled screen

void oled_clear(void); //clearing the screen

void oled_draw_string(uint8_t x, uint8_t page, const char *str); //drawing text at specific positions

//UART

void USART_Init(unsigned long BAUDRATE) { //setting up the serial plotter

uint16_t ubrr_value = (F_CPU / (16UL * BAUDRATE)) - 1;

UCSR0B = (1 << RXEN0) | (1 << TXEN0);

UCSR0C = (1 << UCSZ00) | (1 << UCSZ01);

UBRR0L = ubrr_value;

UBRR0H = (ubrr_value >> 8);

}

void USART_TxChar(char data) { //sending single characters to serial plotter

UDR0 = data;

while (!(UCSR0A & (1<<UDRE0)));

}

void USART_SendString(char *str) { //sending a full string over to serial

int i = 0;

while (str[i] != 0) {

USART_TxChar(str[i]);

i++;

}

}

//I2C

void i2c_init(void) { //preparing the i2c pins

TWSR0 = 0x00;

TWBR0 = 0x48; //100khz

TWCR0 = (1 << TWEN);

}

void i2c_start(void) { //starting conversations with device

TWCR0 = (1 << TWSTA) | (1 << TWEN) | (1 << TWINT);

while (!(TWCR0 & (1 << TWINT)));

}

void i2c_stop(void) { //ending conversations

TWCR0 = (1 << TWSTO) | (1 << TWEN) | (1 << TWINT);

_delay_us(10);

}

void i2c_write(uint8_t data) { //sending byte of data to i2c

TWDR0 = data;

TWCR0 = (1 << TWEN) | (1 << TWINT);

while (!(TWCR0 & (1 << TWINT)));

}

uint8_t i2c_read_ack(void) { //reading byte and acknowledges it

TWCR0 = (1 << TWEN) | (1 << TWINT) | (1 << TWEA);

while (!(TWCR0 & (1 << TWINT)));

return TWDR0;

}

uint8_t i2c_read_nack(void) { //reading byte without acknowledging it

TWCR0 = (1 << TWEN) | (1 << TWINT);

while (!(TWCR0 & (1 << TWINT)));

return TWDR0;

}

//SSD1306 (OLED) functions

#define OLED_ADDR 0x3C

static void oled_cmd(uint8_t c){ //sending command to screen

i2c_start();

i2c_write(OLED_ADDR<<1);

i2c_write(0x00);

i2c_write(c); i2c_stop();

}

static void oled_data(uint8_t d){ //sending pixel to screen

i2c_start();

i2c_write(OLED_ADDR<<1);

i2c_write(0x40);

i2c_write(d);

i2c_stop();

}

static void oled_setpos(uint8_t col,uint8_t page){ //moving cursor to specific pixel location

oled_cmd(0xB0|page);

oled_cmd(0x00|(col&0x0F));

oled_cmd(0x10|(col>>4));

}

const char ssd1306oled_font[][6] PROGMEM = { //data font for letters

{0x00, 0x00, 0x00, 0x00, 0x00, 0x00}, // sp

{0x00, 0x00, 0x00, 0x2f, 0x00, 0x00}, // !

{0x00, 0x00, 0x07, 0x00, 0x07, 0x00}, // "

{0x00, 0x14, 0x7f, 0x14, 0x7f, 0x14}, // #

{0x00, 0x24, 0x2a, 0x7f, 0x2a, 0x12}, // $

{0x00, 0x62, 0x64, 0x08, 0x13, 0x23}, // %

{0x00, 0x36, 0x49, 0x55, 0x22, 0x50}, // &

{0x00, 0x00, 0x05, 0x03, 0x00, 0x00}, // '

{0x00, 0x00, 0x1c, 0x22, 0x41, 0x00}, // (

{0x00, 0x00, 0x41, 0x22, 0x1c, 0x00}, // )

{0x00, 0x14, 0x08, 0x3E, 0x08, 0x14}, // *

{0x00, 0x08, 0x08, 0x3E, 0x08, 0x08}, // +

{0x00, 0x00, 0x00, 0xA0, 0x60, 0x00}, // ,

{0x00, 0x08, 0x08, 0x08, 0x08, 0x08}, // -

{0x00, 0x00, 0x60, 0x60, 0x00, 0x00}, // .

{0x00, 0x20, 0x10, 0x08, 0x04, 0x02}, // /

{0x00, 0x3E, 0x51, 0x49, 0x45, 0x3E}, // 0

{0x00, 0x00, 0x42, 0x7F, 0x40, 0x00}, // 1

{0x00, 0x42, 0x61, 0x51, 0x49, 0x46}, // 2

{0x00, 0x21, 0x41, 0x45, 0x4B, 0x31}, // 3

{0x00, 0x18, 0x14, 0x12, 0x7F, 0x10}, // 4

{0x00, 0x27, 0x45, 0x45, 0x45, 0x39}, // 5

{0x00, 0x3C, 0x4A, 0x49, 0x49, 0x30}, // 6

{0x00, 0x01, 0x71, 0x09, 0x05, 0x03}, // 7

{0x00, 0x36, 0x49, 0x49, 0x49, 0x36}, // 8

{0x00, 0x06, 0x49, 0x49, 0x29, 0x1E}, // 9

{0x00, 0x00, 0x36, 0x36, 0x00, 0x00}, // :

{0x00, 0x00, 0x56, 0x36, 0x00, 0x00}, // ;

{0x00, 0x08, 0x14, 0x22, 0x41, 0x00}, // <

{0x00, 0x14, 0x14, 0x14, 0x14, 0x14}, // =

{0x00, 0x00, 0x41, 0x22, 0x14, 0x08}, // >

{0x00, 0x02, 0x01, 0x51, 0x09, 0x06}, // ?

{0x00, 0x32, 0x49, 0x59, 0x51, 0x3E}, // @

{0x00, 0x7C, 0x12, 0x11, 0x12, 0x7C}, // A

{0x00, 0x7F, 0x49, 0x49, 0x49, 0x36}, // B

{0x00, 0x3E, 0x41, 0x41, 0x41, 0x22}, // C

{0x00, 0x7F, 0x41, 0x41, 0x22, 0x1C}, // D

{0x00, 0x7F, 0x49, 0x49, 0x49, 0x41}, // E

{0x00, 0x7F, 0x09, 0x09, 0x09, 0x01}, // F

{0x00, 0x3E, 0x41, 0x49, 0x49, 0x7A}, // G

{0x00, 0x7F, 0x08, 0x08, 0x08, 0x7F}, // H

{0x00, 0x00, 0x41, 0x7F, 0x41, 0x00}, // I

{0x00, 0x20, 0x40, 0x41, 0x3F, 0x01}, // J

{0x00, 0x7F, 0x08, 0x14, 0x22, 0x41}, // K

{0x00, 0x7F, 0x40, 0x40, 0x40, 0x40}, // L

{0x00, 0x7F, 0x02, 0x0C, 0x02, 0x7F}, // M

{0x00, 0x7F, 0x04, 0x08, 0x10, 0x7F}, // N

{0x00, 0x3E, 0x41, 0x41, 0x41, 0x3E}, // O

{0x00, 0x7F, 0x09, 0x09, 0x09, 0x06}, // P

{0x00, 0x3E, 0x41, 0x51, 0x21, 0x5E}, // Q

{0x00, 0x7F, 0x09, 0x19, 0x29, 0x46}, // R

{0x00, 0x46, 0x49, 0x49, 0x49, 0x31}, // S

{0x00, 0x01, 0x01, 0x7F, 0x01, 0x01}, // T

{0x00, 0x3F, 0x40, 0x40, 0x40, 0x3F}, // U

{0x00, 0x1F, 0x20, 0x40, 0x20, 0x1F}, // V

{0x00, 0x3F, 0x40, 0x38, 0x40, 0x3F}, // W

{0x00, 0x63, 0x14, 0x08, 0x14, 0x63}, // X

{0x00, 0x07, 0x08, 0x70, 0x08, 0x07}, // Y

{0x00, 0x61, 0x51, 0x49, 0x45, 0x43}, // Z

{0x00, 0x00, 0x7F, 0x41, 0x41, 0x00}, // [

{0x00, 0x55, 0x2A, 0x55, 0x2A, 0x55}, // backslash

{0x00, 0x00, 0x41, 0x41, 0x7F, 0x00}, // ]

{0x00, 0x04, 0x02, 0x01, 0x02, 0x04}, // ^

{0x00, 0x40, 0x40, 0x40, 0x40, 0x40}, // _

{0x00, 0x00, 0x01, 0x02, 0x04, 0x00}, // '

{0x00, 0x20, 0x54, 0x54, 0x54, 0x78}, // a

{0x00, 0x7F, 0x48, 0x44, 0x44, 0x38}, // b

{0x00, 0x38, 0x44, 0x44, 0x44, 0x20}, // c

{0x00, 0x38, 0x44, 0x44, 0x48, 0x7F}, // d

{0x00, 0x38, 0x54, 0x54, 0x54, 0x18}, // e

{0x00, 0x08, 0x7E, 0x09, 0x01, 0x02}, // f

{0x00, 0x18, 0xA4, 0xA4, 0xA4, 0x7C}, // g

{0x00, 0x7F, 0x08, 0x04, 0x04, 0x78}, // h

{0x00, 0x00, 0x44, 0x7D, 0x40, 0x00}, // i

{0x00, 0x40, 0x80, 0x84, 0x7D, 0x00}, // j

{0x00, 0x7F, 0x10, 0x28, 0x44, 0x00}, // k

{0x00, 0x00, 0x41, 0x7F, 0x40, 0x00}, // l

{0x00, 0x7C, 0x04, 0x18, 0x04, 0x78}, // m

{0x00, 0x7C, 0x08, 0x04, 0x04, 0x78}, // n

{0x00, 0x38, 0x44, 0x44, 0x44, 0x38}, // o

{0x00, 0xFC, 0x24, 0x24, 0x24, 0x18}, // p

{0x00, 0x18, 0x24, 0x24, 0x18, 0xFC}, // q

{0x00, 0x7C, 0x08, 0x04, 0x04, 0x08}, // r

{0x00, 0x48, 0x54, 0x54, 0x54, 0x20}, // s

{0x00, 0x04, 0x3F, 0x44, 0x40, 0x20}, // t

{0x00, 0x3C, 0x40, 0x40, 0x20, 0x7C}, // u

{0x00, 0x1C, 0x20, 0x40, 0x20, 0x1C}, // v

{0x00, 0x3C, 0x40, 0x30, 0x40, 0x3C}, // w

{0x00, 0x44, 0x28, 0x10, 0x28, 0x44}, // x

{0x00, 0x1C, 0xA0, 0xA0, 0xA0, 0x7C}, // y

{0x00, 0x44, 0x64, 0x54, 0x4C, 0x44}, // z

{0x00, 0x00, 0x08, 0x77, 0x41, 0x00}, // {

{0x00, 0x00, 0x00, 0x63, 0x00, 0x00}, // Â¦

{0x00, 0x00, 0x41, 0x77, 0x08, 0x00}, // }

{0x00, 0x08, 0x04, 0x08, 0x08, 0x04}  // ~

};

static void oled_char(char c) { //drawing single character on the screen

if (c < 0x20 || c > 0x7E) { //valid ASCII range

c = '?'; //replacING invalid with '?'

}

uint8_t idx = c - 0x20;  //index into font array

for (uint8_t i = 0; i < 6; i++) {

oled_data(pgm_read_byte(&ssd1306oled_font[idx][i])); //send each column

}

oled_data(0x00); //spacing column after character

}

static void oled_string(const char* s){ //drawing full string of characters

while(*s) oled_char(*s++);

}

void oled_clear(void){ //filling screen with black pixels

for(uint8_t p=0;p<8;p++){

oled_setpos(0,p);

for(uint8_t c=0;c<128;c++)

oled_data(0);

}

}

void oled_init(void){ //initializing screen with startup commands

const uint8_t seq[] = {

0xAE,

0xD5,

0x80,

0xA8,

0x3F,

0xD3,

0x00,

0x40,

0x8D,

0x14,

0x20,

0x00,

0xA1,

0xC8,

0xDA,

0x12,

0x81,

0xCF,

0xD9,

0xF1,

0xDB,

0x40,

0xA4,

0xA6,

0xAF

};

_delay_ms(100);

for(uint8_t i=0;i<sizeof(seq);i++)

oled_cmd(seq[i]);

oled_clear();

}

//OLED helpers

#define VAL_COL  64

//BMI160

#define BMI160_ADDR 0x69 //connected to vcc

int16_t accData[3], gyrData[3]; //storing accelerometer array

float pitch = 0.0f, roll = 0.0f, yaw = 0.0f, dt = 0.01; //holding calculated angles

void bmi160_write(uint8_t reg, uint8_t data) { //sending commands to sensor

i2c_start();

i2c_write((BMI160_ADDR << 1));

i2c_write(reg);

i2c_write(data);

i2c_stop();

}

void bmi160_read_bytes(uint8_t reg, uint8_t *buf, uint8_t len) { //reading data from sensor

i2c_start();

i2c_write((BMI160_ADDR << 1));

i2c_write(reg);

i2c_start();

i2c_write((BMI160_ADDR << 1) | 1);

for (uint8_t i = 0; i < len; i++) {

buf[i] = (i == len - 1) ? i2c_read_nack() : i2c_read_ack();

}

i2c_stop();

}

void bmi160_init(void) { //configuring the sensor

//reset to ensure clean configuration

bmi160_write(0x7E, 0xB6);

_delay_ms(50);

//configuring accelerometer (100hz ODR, +-2g)

bmi160_write(0x40, 0x2A);  //ODR 100Hz, normal mode

bmi160_write(0x41, 0x03);  //+-2g range

//configuring gyroscope (100hz ODR, +-2000dps)

bmi160_write(0x42, 0x1A); // ODR 100Hz, Normal mode

bmi160_write(0x43, 0x00); //+-2000dps range

//powering on sensors

bmi160_write(0x7E, 0x11); //accel normal mode

_delay_ms(50);

bmi160_write(0x7E, 0x15); //gyro normal mode

_delay_ms(50);

}

void bmi160_read_raw(void) { //reading accelerometer and gyro data to accData/gyrData

uint8_t data[12];

bmi160_read_bytes(0x0C, data, 12);

for (int i = 0; i < 3; i++) {

gyrData[i] = (int16_t)((data[i * 2 + 1] << 8) | data[i * 2]);

accData[i] = (int16_t)((data[i * 2 + 7] << 8) | data[i * 2 + 6]);

}

}

void complementary_filter(void) { //converting raw accelometer data to g forces

//converting raw accelerometer data to g's

float ax = accData[0] / ACCEL_SCALE;

float ay = accData[1] / ACCEL_SCALE;

float az = accData[2] / ACCEL_SCALE;

//calculating angles from accelerometer

float accPitch = atan2f(ay, az) * 180 / M_PI; //pitch angle from gravity

float accRoll = atan2f(ax, az) * 180 / M_PI; //roll angle from gravity

//gyro data to dps and integrating

pitch += (gyrData[0] / GYRO_SCALE) * dt; //updating pitch using gyro

roll -= (gyrData[1] / GYRO_SCALE) * dt; //same for roll

yaw += (gyrData[2] / GYRO_SCALE) * dt; //same for yaw

//mixing accelerometer and gyro data

pitch = pitch * 0.98f + accPitch * 0.02f; //98% for gyro, 2% for acceleormeter

roll = roll * 0.98f + accRoll * 0.02f; //same for roll

}

//MAIN

int main(void) {

char buffer[32]; //temporary string to hold numbers

i2c_init(); //i2c communication

USART_Init(9600);

oled_init(); //turning on screen

oled_clear(); //clearing the screen

//static labels once during setup

oled_setpos(0, 0); //moving cursor to top left

oled_string("Pitch:"); //writing pitch on the screen

oled_setpos(0, 2); //moving cursor to middle left

oled_string("Roll:"); //writing roll

oled_setpos(0, 4); //moving cursor to bottom left

oled_string("Yaw:"); //writing yaw

_delay_ms(500); //waiting for stability

bmi160_init(); //configuring the motion sensor

while (1) {

bmi160_read_raw(); //looping forever

complementary_filter(); //calculating pitch/roll/yaw

//UART output for pitch, roll and yaw

USART_SendString("Pitch: ");

dtostrf(pitch, 6, 2, buffer); //converting pitch into a string using 6 characters and 2 decimals

USART_SendString(buffer); //sending pitch value using a buffer

USART_SendString("\t\t");

USART_SendString("Roll: ");

dtostrf(roll, 6, 2, buffer); //same logic for the rest of these

USART_SendString(buffer);

USART_SendString("\t\t");

USART_SendString("Yaw: ");

dtostrf(yaw, 6, 2, buffer);

USART_SendString(buffer);

USART_SendString("\r\n");

//sending accelerometer data

USART_SendString("AccX: ");

dtostrf(accData[0]/ACCEL_SCALE, 6, 2, buffer);

USART_SendString(buffer);

USART_SendString("\t\t");

USART_SendString("AccY: ");

dtostrf(accData[1]/ACCEL_SCALE, 6, 2, buffer);

USART_SendString(buffer);

USART_SendString("\t\t");

USART_SendString("AccZ: ");

dtostrf(accData[2]/ACCEL_SCALE, 6, 2, buffer);

USART_SendString(buffer);

USART_SendString("\r\n");

//sending Gyro data

USART_SendString("GyroX: ");

dtostrf(gyrData[0]/GYRO_SCALE, 6, 2, buffer);

USART_SendString(buffer);

USART_SendString("\t\t");

USART_SendString("GyroY: ");

dtostrf(gyrData[1]/GYRO_SCALE, 6, 2, buffer);

USART_SendString(buffer);

USART_SendString("\t\t");

USART_SendString("GyroZ: ");

dtostrf(gyrData[2]/GYRO_SCALE, 6, 2, buffer);

USART_SendString(buffer);

USART_SendString("\r\n");

_delay_ms(100);

}

}

- SCHEMATIC

- SCREENSHOTS OF EACH TASK OUTPUT (

TASK 1

Accelerometer on terminal window:

Gyro:

Pitch, roll, yaw:

TASK 2

Accelerometer on serial plotter:

Gyro:

Pitch, roll, yaw:

- SCREENSHOT OF DEMO

- VIDEO LINK FOR DEMOS

Task (serial plotter):

https://youtu.be/q7OJiuuBne4

Task (terminal):

https://youtu.be/JJvWWR_0mi0

- GITHUB LINK OF THIS DA

Tasks c code: https://github.com/CaFu0320/submission_da/tree/main/DesignAssignments/DA7/DA_7v2/DA_7v2

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Carlos Funes