CPE301 – SPRING 2024

# Design Assignment 2

Student Name: Carlos Funes

Student #: 5007995832

Student Email: funesc2@unlv.nevada.edu

Primary Github address: https://github.com/CaFu0320

Directory:https://github.com/CaFu0320/submission_da/tree/main/DesignAssignments/DesignAssignment2

Video Playlist: https://www.youtube.com/playlist?list=PLa3xS6s509hiXVF-aiEOqfY5oCL0WiYr7

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- ATMEGA328PB MINI BOARD

- FEMALE TO FEMALE CABLE

- MALE TO MALE CABLE

- LOGIC ANALYZER

- ARDUINO SHIELD

- C CODE TASK 2/A

/*

* main.c

*

* Created: 3/8/2025 3:15:20 PM

*  Author: Carlos Funes

*/

#define F_CPU 8000000UL

#include <avr/io.h>

#include <util/delay.h>

void delay_150ms(void) {

volatile uint16_t i = 0;

volatile uint16_t j = 0;

while(i < 660) {       //outer loop

j = 0;

while(j < 200) {   //inner loop

j++;

}

i++;

}

} //this loop creates 660*200 = 132k iterations

int main(void) {            // main function entry point

DDRC &= ~(1 << 1); //set pc1 for input

PORTC |= (1 << 1); //enable pull-up resistor for pc1

DDRB |= (1 << 5); //set pb5 for output

PORTB &= ~(1 << 5); //make sure led on pb5 is initially off

while(1) {            // start infinite loop

if (!(PINC & (1 << 1))) {  // check if button at pc1 is pressed (active low)

PORTB |= (1 << 5);     // turn on led on pb5

uint8_t count = 10;    // we want a total delay of 1.5 seconds (10 x 150 ms)

while(count--) {       // loop 10 times

delay_150ms();     // delay occurs here (~150 ms delay)

}

PORTB &= ~(1 << 5);    // turn off led on pb5 after delay

} else {

PORTB &= ~(1 << 5);    // ensure led remains off when button is not pressed

}

}

return 0;               // end of main function

}

ASSEMBLY PART 1

;

; DA2PART1ASSEMBLY.asm

;

; Created: 3/20/2025 4:22:28 PM

; Author : Carlos Funes

;

.INCLUDE <M328PBDEF.INC>

.ORG 0 ;RESET LOCATION

JMP MAIN ;JUMPING TO MAIN

DELAY_150MS:        ;DELAY SUBROUTINE

LDI R17, 0x30 ;THIS IS OUTER LOOP

L1:

LDI R18, 0x40 ;MIDDLE LOOP

L2:

LDI R19, 0x60 ;INNER LOOP

L3:

DEC	R19 ;DECREMENTING R19

CPI R19, 0 ;

BRNE L3 ;BRANCH TO L3 IF R19 /= 0

DEC R18 ;DECREMENTING R18

CPI R18, 0

BRNE L2 ;BRANCH TO L2 IF R18 /= 0

DEC R17 ;DECREMENTING R17

CPI R17, 0

BRNE L1 ;BRANCH TO L1 IF R17 /= 0

RET	;RETURN TO THE PROGRAM

MAIN:

LDI R20, HIGH(RAMEND) ;STARTING STACK POINTER

OUT SPH, R20

LDI R20, LOW(RAMEND)

OUT SPL, R20

SBI DDRB, 5 ;PORTB5 AS OUTPUT

CBI PORTB, 5 ;PORTB5 INITIALIZED TO OFF

CBI DDRC, 1 ; PORTC1 AS INPUT

SBI PORTC, 1 ;ENABLING PULL UP RESISTORS FOR PUSHBUTTON

WHILE:

SBIC PINC, 1 ;CHECKING IF PUSHBUTTON PC1 IS PRESSED

RJMP CHECKER ;IF BUTTON IS RELEASED

RJMP WHILE ;LOOPING FOREVER IF BUTTON IS NOT PRESSED

CHECKER:

SBIS PINC, 1 ;CHECK IF BUTTON IS NOT PRESSED

RJMP LED_ON ;IF BUTTON IS PRESSED, JUMP TO LED

RJMP CHECKER ;ONCE BUTTON IS PRESSED, STOP THE LOOP

LED_ON:

SBI PORTB, 5 ;TURN ON PB5 LED

LDI R16, 0x14 ;1.5 SECOND DELAY

L0:

CALL DELAY_150MS ;CALLING DELAY SUBROUTINE

DEC R16 ;DECREMENT R16

CPI R16, 0 ;CHECK IF R16 = 0

BRNE L0 ;CONTINUE TO LOOP IF R16 /= 0

CBI PORTB, 5 ;AFTER 1.5 SECONDS, TURN OFF LED

RJMP WHILE ;JUMPING BACK TO THE MAIN LOOP

Insert initial code here

- C CODE TASK 3/A

/*

* main.c

*

* Created: 3/20/2025 3:15:20 PM

*  Author: Carlos Funes

*/

#define F_CPU 8000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

void delay_150ms(void) {

volatile uint16_t i = 0;

volatile uint16_t j = 0;

while(i < 660) {         // outer loop remains at 660 iterations

j = 0;

while(j < 200) {    // inner loop: 200 iterations

j++;

}

i++;

}

}

ISR(INT0_vect) {

// Interrupt-based handling: turn on LED and delay for 3 sec.

PORTB |= (1 << 5);            // Turn on LED at PB5

for(int k = 0; k < 10; k++){

delay_150ms();

}                // Delay 3 seconds (blocking delay)

PORTB &= ~(1 << 5);           // Turn off LED

}

int main(void) {

// Configure LED output

DDRB |= (1 << 5);             // Set PB5 as output

PORTB &= ~(1 << 5);           // Make sure LED is initially off

// Configure switch on INT0 (PD2)

DDRD &= ~(1 << 2);            // Set PD2 as input

PORTD |= (1 << 2);            // Enable internal pull-up on PD2

// Configure INT0 to trigger on a rising edge (active high)

EICRA |= (1 << ISC01) | (1 << ISC00);  // Rising edge trigger for INT0

EIMSK |= (1 << INT0);                  // Enable external interrupt INT0

sei();                          // Enable global interrupts

while(1) {

// this is just to keep the microcontroller running

}

return 0;

}

ASSEMBLY PART 2

;

; DA2PART2ASSEMBLY.asm

;

; Created: 3/20/2025 5:06:26 PM

; Author : Carlos Funes

;

; Replace with your application code

.INCLUDE <M328PBDEF.INC>

.ORG 0 ;RESET LOCATION

JMP MAIN ;JUMPING TO MAIN

.ORG 0x02 ;INT0 VECTOR

JMP ISR_INT0VECTOR

DELAY_150MS:        ;DELAY SUBROUTINE

LDI R17, 0x30 ;THIS IS OUTER LOOP

L1:

LDI R18, 0x40 ;MIDDLE LOOP

L2:

LDI R19, 0x60 ;INNER LOOP

L3:

DEC	R19 ;DECREMENTING R19

CPI R19, 0 ;

BRNE L3 ;BRANCH TO L3 IF R19 /= 0

DEC R18 ;DECREMENTING R18

CPI R18, 0

BRNE L2 ;BRANCH TO L2 IF R18 /= 0

DEC R17 ;DECREMENTING R17

CPI R17, 0

BRNE L1 ;BRANCH TO L1 IF R17 /= 0

RET	;RETURN TO THE PROGRAM

MAIN:

LDI R20, HIGH(RAMEND) ;STARTING STACK POINTER

OUT SPH, R20

LDI R20, LOW(RAMEND)

OUT SPL, R20

LDI R20, 0x03 ;MAKING INT0 BE TRIGGERED AT RISING EDGE, LAST 2 BITS MAKE THIS HAPPEN

STS EICRA, R20 ;STORING R20 INTO EICRA

SBI EIMSK, INT0 ;ENABLING INT0 INTERRUPT

SEI

SBI DDRB, 5 ;PORTB5 AS OUTPUT

CBI PORTB, 5 ;PORTB5 INITIALIZED TO OFF

CBI DDRD, 2 ; PORTD2 AS INPUT

SBI PORTD, 2 ;ENABLING PULL UP RESISTORS FOR PUSHBUTTON

WHILE:

CBI PORTB, 5 ;LED IS OFF WHILE IN THE LOOP

RJMP WHILE ;LOOP FOREVER

ISR_INT0VECTOR:

SBI PORTB, 5 ;TURN ON PB5 LED

LDI R16, 0x14 ;1.5 SECOND DELAY

LJ:

CALL DELAY_150MS ;CALLING DELAY SUBROUTINE

DEC R16 ;DECREMENT R16

CPI R16, 0 ;CHECK IF R16 = 0

BRNE LJ ;CONTINUE TO LOOP IF R16 /= 0

CBI PORTB, 5 ;AFTER 1.5 SECONDS, TURN OFF LED

RETI ;RETURNING FROM INTERRUPT

- C CODE TASK 2 AND 3

/*

* DA2PART3CCODE.c

*

* Created: 3/20/2025 4:13:42 PM

* Author : Carlos Funes

*/

//Question 3)

#define F_CPU 8000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

void delay_150ms(void) {

volatile uint16_t i = 0;

volatile uint16_t j = 0;

while(i < 660) {         //outer loop remains at 660 iterations

j = 0;

while(j < 200) {    //inner loop: 200 * 20 = 4000 iterations

j++;

}

i++;

}

}

ISR(INT0_vect) {

//interrupt-based handling: turn on LED and delay for 3 sec.

PORTB |= (1 << 5);            //turn on LED at PB5

for(int k = 0; k < 10; k++ ){

delay_150ms();

}

//delay 3 seconds (blocking delay)

PORTB &= ~(1 << 5);           //turn off LED

}

int main(void) {

DDRB |= (1 << 5);             //set PB5 as output

PORTB &= ~(1 << 5);           //make sure LED is initially off

DDRC &= ~(1 << 1); //set pc1 for input

PORTC |= (1 << 1); //enable pull-up resistor for pc1

DDRD &= ~(1 << 2);            //set PD2 as input

PORTD |= (1 << 2);            //enable internal pull-up on PD2

EICRA |= (1 << ISC01) | (1 << ISC00);  //rising edge trigger for INT0

EIMSK |= (1 << INT0);                  //enable external interrupt INT0

sei();                          //enable global interrupts

while(1) {            //start infinite loop

if (!(PINC & (1 << 1))) {  //check if button at pc1 is pressed (active low)

PORTB |= (1 << 5);     //turn on led on pb5

uint8_t count = 10;    //we want a total delay of 1.5 seconds (10 x 150 ms)

while(count--) {       //loop 10 times

delay_150ms();     //delay occurs here (~150 ms delay)

}

PORTB &= ~(1 << 5);    //turn off led on pb5 after delay

} else {

PORTB &= ~(1 << 5);    //ensure led remains off when button is not pressed

}

}

return 0;

}

ASSEMBLY PART 3

;

; DA2PART3ASSEMBLY.asm

;

; Created: 3/20/2025 5:16:02 PM

; Author : Carlos Funes

;

.INCLUDE <M328PBDEF.INC>

.ORG 0 ;RESET LOCATION

JMP MAIN ;JUMPING TO MAIN

.ORG 0x02 ;INT0 VECTOR

JMP ISR_INT0VECTOR

DELAY_150MS:

LDI R17, 0x30 ;THIS IS OUTER LOOP

L1:

LDI R18, 0x40 ;MIDDLE LOOP

L2:

LDI R19, 0x60 ;INNER LOOP

L3:

DEC	R19 ;DECREMENTING R19

CPI R19, 0 ;

BRNE L3 ;BRANCH TO L3 IF R19 /= 0

DEC R18 ;DECREMENTING R18

CPI R18, 0

BRNE L2 ;BRANCH TO L2 IF R18 /= 0

DEC R17 ;DECREMENTING R17

CPI R17, 0

BRNE L1 ;BRANCH TO L1 IF R17 /= 0

RET	;RETURN TO THE PROGRAM

MAIN:

LDI R20, HIGH(RAMEND) ;STARTING STACK POINTER

OUT SPH, R20

LDI R20, LOW(RAMEND)

OUT SPL, R20

LDI R20, 0x03 ;MAKING INT0 BE TRIGGERED AT RISING EDGE, LAST 2 BITS MAKE THIS HAPPEN

STS EICRA, R20 ;STORING R20 INTO EICRA

SBI EIMSK, INT0 ;ENABLING INT0 INTERRUPT

SEI

SBI DDRB, 5 ;PORTB5 AS OUTPUT

CBI PORTB, 5 ;PORTB5 INITIALIZED TO OFF

CBI DDRD, 2 ; PORTD2 AS INPUT

SBI PORTD, 2 ;ENABLING PULL UP RESISTORS FOR PUSHBUTTON

CBI DDRC, 1 ; PORTC1 AS INPUT

SBI PORTC, 1 ;ENABLING PULL UP RESISTORS FOR PUSHBUTTON

LDI R20,(1<<ISC01)|(1<<ISC00) ;SETTING INT0 RISING EDGE TRIGGER

STS EICRA,R20 ;STORING TO INT CONTROL REGISTER

SBI EIMSK,INT0 ;ENABLING INT0 INTERRUPT

SEI ;GLOBAL INTERRUPT

POLL_LOOP:

SBIC PINC,1 ;CHECKING IF BUTTON PC1 PRESSED

RJMP POLL_LOOP ;KEEP POLLING IF NOT PRESSED

CALL DELAY_150MS ;DEBOUNCE DELAY 0.15 SEC

WAIT_RELEASE:

SBIS PINC,1 ;CHECK IF BUTTON RELEASED

RJMP WAIT_RELEASE ;WAIT UNTIL BUTTON RELEASED

SBI PORTB,5 ;TURN ON LED

LDI R16,20 ;SET 1.5 SEC DELAY

DELAY_1_5S:

CALL DELAY_150MS ;CALLING 0.15 SEC DELAY

DEC R16 ;DECREMENT COUNTER

BRNE DELAY_1_5S ;REPEAT 20 TIMES

CBI PORTB,5 ;TURN OFF LED

RJMP POLL_LOOP ;RETURN TO POLLING LOOP

ISR_INT0VECTOR:

SBI PORTB,5 ;TURN ON LED ON INTERRUPT

PUSH R16 ;SAVE REGISTER STATE

LDI R16,20 ;SET 3 SEC DELAY

DELAY_3S:

CALL DELAY_150MS ;CALLING 0.15 SEC DELAY

DEC R16 ;DECREMENT COUNTER

BRNE DELAY_3S ;REPEATING 20 TIMES

CBI PORTB,5 ;TURN OFF LED

POP R16 ;RESTORING REGISTER STATE

RETI ;RETURN FROM INTERRUPT

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (LOGIC ANALYZER)

Task 2:

Task 3:

Task 2 & 3 Polling and Interrupt method

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Task 2:

Task 3:

- VIDEO LINKS OF EACH DEMO

Task 2:

https://www.youtube.com/watch?v=6eriUlZqST4&list=PLa3xS6s509hiXVF-aiEOqfY5oCL0WiYr7&index=3&ab_channel=CarlosGordfgg

Task 3:

https://www.youtube.com/watch?v=prNirN60sFA&list=PLa3xS6s509hiXVF-aiEOqfY5oCL0WiYr7&index=1&ab_channel=CarlosGordfgg

Task 2 & 3 Polling and Interrupt method:

https://www.youtube.com/watch?v=uCc1aDGF9xg&list=PLa3xS6s509hiXVF-aiEOqfY5oCL0WiYr7&index=3&ab_channel=CarlosGordfgg

- GITHUB LINK OF THIS DA

Task 2 c code: https://github.com/CaFu0320/submission_da/tree/main/DesignAssignments/DesignAssignment2/DA2PART1CCODE

Task 2 assembly code: https://github.com/CaFu0320/submission_da/tree/main/DesignAssignments/DesignAssignment2/DA2PART1ASSEMBLY

Task 3 c code: https://github.com/CaFu0320/submission_da/tree/main/DesignAssignments/DesignAssignment2/DA2PART2CCODE

Task 3 assembly code:

https://github.com/CaFu0320/submission_da/tree/main/DesignAssignments/DesignAssignment2/DA2PART2ASSEMBLY

Task 2 and 3 c code: https://github.com/CaFu0320/submission_da/tree/main/DesignAssignments/DesignAssignment2/DA2PART3CCODE

Task 2 and 3 assembly:

https://github.com/CaFu0320/submission_da/tree/main/DesignAssignments/DesignAssignment2/DA2PART3ASSEMBLY

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT